SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	:	SULPHAMIC ACID

Product Number	:	CS6874-56

Brand	:	Prime Chemicals

Index-No.	:	016-026-00-0

REACH No.	:	A registration number is not available for this substance as the substance

or its uses are exempted from registration, the annual tonnage does not require a registration or the registration is envisaged for a later registration deadline.

CAS-No.	:	5329-14-6

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Skin irritation (Category 2), H315 Eye irritation (Category 2), H319

Chronic aquatic toxicity (Category 3), H412

For the full text of the H-Statements mentioned in this Section, see Section 16.

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

R52/53

Xi	Irritant	R36/38

For the full text of the R-phrases mentioned in this Section, see Section 16.

# Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram	 Signal word	Warning

Hazard statement(s)

H315	Causes skin irritation.

H319	Causes serious eye irritation.

H412	Harmful to aquatic life with long lasting effects.

Precautionary statement(s)

P273	Avoid release to the environment.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

Supplemental Hazard Statements

  - Other hazards - none

none

# SECTION 3: Composition/information on ingredients

| 3.1 | Substances Synonyms : | Amidosulfonic acid |
| --- | --- | --- |
|  | Formula : Molecular Weight : CAS-No. : | H3NO3S 97.09 g/mol 5329-14-6 |
|  | EC-No. : | 226-218-8 |
|  | Index-No. : | 016-026-00-0 |

Hazardous ingredients according to Regulation (EC) No 1272/2008

| Component | Classification | Concentration |
| --- | --- | --- |
| Sulphamidic acid |  |  |
| CAS-No. 5329-14-6 EC-No. 226-218-8 Index-No. 016-026-00-0 | Skin Irrit. 2; Eye Irrit. 2; Aquatic Chronic 3; H315, H319, H412 | <= 100 % |

Hazardous ingredients according to Directive 1999/45/EC

| Component | Classification | Concentration |
| --- | --- | --- |
| Sulphamidic acid |  |  |
| CAS-No. 5329-14-6 EC-No. 226-218-8 Index-No. 016-026-00-0 | Xi, R36/38 - R52/53 | <= 100 % |

For the full text of the H-Statements and R-Phrases mentioned in this Section, see Section 16

# SECTION 4: First aid measures

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

no data available

# SECTION 5: Firefighting measures

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

nitrogen oxides (NOx), Sulphur oxides

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Avoid breathing dust.

For personal protection see section 8.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

# Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

impervious clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher level protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: powder, crystalline Colour: white

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	1.5 at 10 g/l at 20 °C

    - Melting point/freezing point

    - Initial boiling point and boiling range

205 °C

no data available

    - Flash point	no data available

    - Evapouration rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

no data available

    - Vapour pressure	0.008 hPa at 20 °C

0.025 hPa at 100 °C

    - Vapour density	no data available

    - Relative density	2.151 g/cm3 at 25 °C

    - Water solubility	213 g/l at 20 °C470 g/l at 80 °C

    - Partition coefficient: n- octanol/water

    - Auto-ignition temperature

    - Decomposition temperature

no data available no data available 209 °C -

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

no data available

# SECTION 10: Stability and reactivity

  - Reactivity

no data available

# Chemical stability

Stable under recommended storage conditions.

# Possibility of hazardous reactions

no data available

# Conditions to avoid

no data available

# Incompatible materials

Strong oxidizing agents, Strong bases

# Hazardous decomposition products

Other decomposition products - no data available In the event of fire: see section 5

# SECTION 11: Toxicological information

11.1	Information on toxicological effects Acute toxicity

LD50 Oral - rat - 3,160 mg/kg (OECD Test Guideline 401)

LD50 Oral - mouse - 1,312 mg/kg

Remarks: Behavioral:Excitement. Behavioral:Altered sleep time (including change in righting reflex).

LD50 Oral - guinea pig - 1,050 mg/kg

Remarks: Behavioral:Excitement. Behavioral:Altered sleep time (including change in righting reflex).

Inhalation: no data available Dermal: no data available

# Skin corrosion/irritation

Skin - rabbit

Result: Moderate skin irritation (OECD Test Guideline 404)

Skin - Human

Result: Mild skin irritation

# Serious eye damage/eye irritation

Eyes - rabbit

Result: Moderate eye irritation (OECD Test Guideline 405)

# Respiratory or skin sensitisation

no data available

# Germ cell mutagenicity

no data available Carcinogenicity no data available

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Additional Information

RTECS: Not available

Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., Symptoms and signs of poisoning are:, burning sensation, Cough, wheezing, laryngitis, Shortness of breath, Headache, Nausea, Vomiting, Inhalation may provoke the following symptoms:, spasm, inflammation and edema of the bronchi, spasm, inflammation and edema of the larynx, Aspiration or inhalation may cause chemical pneumonitis.

# SECTION 12: Ecological information

  - Toxicity

Toxicity to fish	static test LC50 - Pimephales promelas (fathead minnow) - 70.3 mg/l - 96 h (OECD Test Guideline 203)

Toxicity to daphnia and other aquatic invertebrates

Remarks: no data available

Toxicity to algae	Remarks: no data available

# Persistence and degradability

Biodegradability	Result: - Not readily biodegradable.

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

# Other adverse effects

Harmful to aquatic life with long lasting effects.

# SECTION 13: Disposal considerations

13.1	Waste treatment methods Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

# Contaminated packaging

Dispose of as unused product.

# SECTION 14: Transport information

| 14.1 | UN number |  |
| --- | --- | --- |
| 14.2 | ADR/RID: 2967 UN proper shipping name | IMDG: 2967 |

IATA: 2967

ADR/RID:	SULPHAMIC ACID IMDG:	SULPHAMIC ACID

IATA:	Sulphamic acid

| 14.3 | Transport hazard class(es) ADR/RID: 8 | IMDG: 8 | IATA: 8 |
| --- | --- | --- | --- |
| 14.4 | Packaging group ADR/RID: III | IMDG: III | IATA: III |
| 14.5 | Environmental hazards ADR/RID: no | IMDG Marine pollutant: no | IATA: no |
| 14.6 | Special precautions for user no data available |  |  |

# SECTION 15: Regulatory information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

# SECTION 16: Other information

Full text of H-Statements referred to under sections 2 and 3.

Aquatic Chronic	Chronic aquatic toxicity Eye Irrit.	Eye irritation

H315	Causes skin irritation.

H319	Causes serious eye irritation.

H412	Harmful to aquatic life with long lasting effects.

Skin Irrit.	Skin irritation

# Full text of R-phrases referred to under sections 2 and 3

Xi	Irritant

R36/38	Irritating to eyes and skin.

R52/53	Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

# Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.