SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	:	Catechol

Product Number	:	CC3304-52

Brand	:	Prime Chemicals

Index-No.	:	604-016-00-4

REACH No.	:	A registration number is not available for this substance as the substance

or its uses are exempted from registration, the annual tonnage does not require a registration or the registration is envisaged for a later registration deadline.

CAS-No.	:	120-80-9

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Acute toxicity, Oral (Category 3), H301 Acute toxicity, Inhalation (Category 4), H332 Acute toxicity, Dermal (Category 3), H311 Skin irritation (Category 2), H315

Serious eye damage (Category 1), H318 Skin sensitisation (Category 1), H317 Germ cell mutagenicity (Category 2), H341

For the full text of the H-Statements mentioned in this Section, see Section 16.

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

| Xn | Harmful | R20/21/22 |
| --- | --- | --- |
| Xi | Irritant | R38, R41 |
|  |  | R43, R68 |

For the full text of the R-phrases mentioned in this Section, see Section 16.

# Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram

Signal word	Danger

Hazard statement(s)

H301 + H311	Toxic if swallowed or in contact with skin

H315	Causes skin irritation.

H317	May cause an allergic skin reaction.

H318	Causes serious eye damage.

H332	Harmful if inhaled.

H341	Suspected of causing genetic defects.

Precautionary statement(s)

P280	Wear protective gloves/ eye protection/ face protection.

P301 + P310	IF SWALLOWED: Immediately call a POISON CENTER or doctor/

physician.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P312	Call a POISON CENTER or doctor/ physician if you feel unwell.

Supplemental Hazard Statements

# Other hazards

Rapidly absorbed through skin.

none

# SECTION 3: Composition/information on ingredients

| 3.1 | Substances Synonyms : | 1,2-Benzenediol |
| --- | --- | --- |
|  |  | Catechol 1,2-Dihydroxybenzene |
|  | Formula : Molecular Weight : CAS-No. : | C6H6O2 110.11 g/mol 120-80-9 |
|  | EC-No. : | 204-427-5 |
|  | Index-No. : | 604-016-00-4 |

Hazardous ingredients according to Regulation (EC) No 1272/2008

| Component | Classification | Concentration |
| --- | --- | --- |
| Pyrocatechol |  |  |
| CAS-No. 120-80-9 EC-No. 204-427-5 Index-No. 604-016-00-4 | Acute Tox. 3; Acute Tox. 4; Acute Tox. 3; Skin Irrit. 2; Eye Dam. 1; Skin Sens. 1; Muta. 2; H301 + H311, H315, H317, H318, H332, H341 | <= 100 % |

Hazardous ingredients according to Directive 1999/45/EC

| Component | Classification | Concentration |
| --- | --- | --- |
| Pyrocatechol |  |  |
| CAS-No. 120-80-9 EC-No. 204-427-5 Index-No. 604-016-00-4 | Xn, R20/21/22 - R38 - R41R43 - R68 | <= 100 % |

For the full text of the H-Statements and R-Phrases mentioned in this Section, see Section 16

# SECTION 4: First aid measures

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

no data available

# SECTION 5: Firefighting measures

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

no data available

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Wear respiratory protection. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

For personal protection see section 8.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Store under inert gas. Air and light sensitive.

# Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

Components with workplace control parameters

| Component | CAS-No. | Value | Control parameters | Basis |
| --- | --- | --- | --- | --- |
| Pyrocatechol | 120-80-9 | TWA | 5 ppm 23 mg/m3 | UK. EH40 WEL - Workplace Exposure Limits |
|  | Remarks | Where no specific short-term exposure limit is listed, a figure three times the long-term exposure should be used |  |  |

  - Exposure controls

Appropriate engineering controls

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

# Personal protective equipment Eye/face protection

Face shield and safety glasses Use equipment for eye protection tested and approved under

appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Complete suit protecting against chemicals, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: powder

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	no data available

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 100 - 103 °C - lit. 245 °C - lit.

    - Flash point	127 °C - closed cup

    - Evapouration rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

Lower explosion limit: 1.97 %(V)

    - Vapour pressure	13 hPa at 118.3 °C

1 hPa at 75 °C

    - Vapour density	no data available

    - Relative density	no data available

    - Water solubility	no data available

    - Partition coefficient: n- octanol/water

    - Auto-ignition temperature

    - Decomposition temperature

log Pow: 0.88

no data available no data available

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

no data available

# SECTION 10: Stability and reactivity

  - Reactivity

no data available

# Chemical stability

Stable under recommended storage conditions.

# Possibility of hazardous reactions

no data available

# Conditions to avoid

no data available

# Incompatible materials

Oxidizing agents

# Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - Carbon oxides Other decomposition products - no data available

In the event of fire: see section 5

# SECTION 11: Toxicological information

11.1	Information on toxicological effects Acute toxicity

LD50 Oral - rat - male - 300 mg/kg Inhalation: no data available

LD50 Dermal - rabbit - male - 800 mg/kg

# Skin corrosion/irritation

no data available

# Serious eye damage/eye irritation

Eyes - rabbit

Result: Risk of serious damage to eyes.

# Respiratory or skin sensitisation

- guinea pig

Result: May cause sensitisation by skin contact.

# Germ cell mutagenicity

In vitro tests showed mutagenic effects

mouse lymphocyte Result: positive

rat - male Result: positive

# Carcinogenicity

This product is or contains a component that has been reported to be possibly carcinogenic based on its IARC, ACGIH, NTP, or EPA classification.

IARC:	2B - Group 2B: Possibly carcinogenic to humans (Pyrocatechol)

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Additional Information

RTECS: UX1050000

Cough, Shortness of breath, Headache, Nausea, Vomiting, To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

# SECTION 12: Ecological information

  - Toxicity

Toxicity to fish	LC50 - Pimephales promelas (fathead minnow) - 3.5 mg/l - 96 h

# Persistence and degradability

Biodegradability	aerobic - Exposure time 28 d

Result: 62 % - Readily biodegradable. (OECD Test Guideline 301B)

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

# Other adverse effects

Toxic to aquatic life. no data available

# SECTION 13: Disposal considerations

13.1	Waste treatment methods Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material

with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

# Contaminated packaging

Dispose of as unused product.

# SECTION 14: Transport information

| 14.1 | UN number |  |
| --- | --- | --- |
| 14.2 | ADR/RID: 2811 UN proper shipping name | IMDG: 2811 |

IATA: 2811

ADR/RID:	TOXIC SOLID, ORGANIC, N.O.S. (Pyrocatechol) IMDG:	TOXIC SOLID, ORGANIC, N.O.S. (Pyrocatechol)

IATA:	Toxic solid, organic, n.o.s. (Pyrocatechol)

| 14.3 | Transport hazard class(es) ADR/RID: 6.1 | IMDG: 6.1 | IATA: 6.1 |
| --- | --- | --- | --- |
| 14.4 | Packaging group ADR/RID: III | IMDG: III | IATA: III |
| 14.5 | Environmental hazards ADR/RID: no | IMDG Marine pollutant: no | IATA: no |
| 14.6 | Special precautions for user no data available |  |  |

# SECTION 15: Regulatory information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

# SECTION 16: Other information

Full text of H-Statements referred to under sections 2 and 3.

Acute Tox.	Acute toxicity

Eye Dam.	Serious eye damage

H301	Toxic if swallowed.

H301 + H311	Toxic if swallowed or in contact with skin H311	Toxic in contact with skin.

H315	Causes skin irritation.

H317	May cause an allergic skin reaction.

H318	Causes serious eye damage.

H332	Harmful if inhaled.

H341	Suspected of causing genetic defects.

Muta.	Germ cell mutagenicity

# Full text of R-phrases referred to under sections 2 and 3

Xn	Harmful

R20/21/22	Harmful by inhalation, in contact with skin and if swallowed. R38	Irritating to skin.

R41	Risk of serious damage to eyes.

R43	May cause sensitisation by skin contact.

R68	Possible risk of irreversible effects.

# Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.