SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifiers

Product name	:	Barium

Product Number	:	CB1200-46

Brand	:	Prime Chemicals

CAS-No.	:	7440-39-3

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# HAZARDS IDENTIFICATION

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP] Substances, which in contact with water, emit flammable gases (Category 2) Skin irritation (Category 2)

Eye irritation (Category 2)

Specific target organ toxicity - single exposure (Category 3)

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

Irritating to eyes, respiratory system and skin. Reacts violently with water, liberating extremely flammable gases.

# Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Pictogram	 Signal word	Danger

Hazard statement(s)

H261	In contact with water releases flammable gases.

H315	Causes skin irritation.

H319	Causes serious eye irritation.

H335	May cause respiratory irritation.

Precautionary statement(s)

P231 + P232	Handle under inert gas. Protect from moisture.

P261	Avoid breathing dust.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P422	Store contents under inert gas.

Supplemental Hazard Statements

none

# According to European Directive 67/548/EEC as amended.

|  | R-phrase(s) R36/37/38 R14/15 | Irritating to eyes, respiratory system and skin. Reacts violently with water, liberating extremely flammable gases. |
| --- | --- | --- |
|  | S-phrase(s) S16 | Keep away from sources of ignition - No smoking. |
|  | S26 S36/37 | In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear suitable protective clothing and gloves. |
|  | S43 | In case of fire, use sand. |
| 2.3 | Other hazards - none |  |

Hazard symbol(s)

# COMPOSITION/INFORMATION ON INGREDIENTS

  - Substances

Formula	:	Ba

Molecular Weight	:	137.33 g/mol

| Component | Concentration |
| --- | --- |
| Barium |  |
| CAS-No. 7440-39-3 EC-No. 231-149-1 | - |

# FIRST AID MEASURES

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

Stomach/intestinal disorders, Nausea, Vomiting, Drowsiness, Dizziness, Gastrointestinal disturbance, Weakness, Tremors, Seizures.

# Indication of any immediate medical attention and special treatment needed

no data available

# FIREFIGHTING MEASURES

  - Extinguishing media  Suitable extinguishing media

Dry powder Carbon dioxide (CO2)

# Unsuitable extinguishing media

Water

# Special hazards arising from the substance or mixture

Barium oxide

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# ACCIDENTAL RELEASE MEASURES

  - Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

# Methods and materials for containment and cleaning up

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Do not flush with water. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# HANDLING AND STORAGE

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.Keep away from sources of ignition

- No smoking.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Never allow product to get in contact with water during storage.

Store under inert gas.

# Specific end uses

no data available

# EXPOSURE CONTROLS/PERSONAL PROTECTION

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

# Body Protection

impervious clothing, Flame retardant protective clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# PHYSICAL AND CHEMICAL PROPERTIES

  - Information on basic physical and chemical properties

    - Appearance	Form: Pieces Colour: grey

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	no data available

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 725 °C - lit. 1,640 °C - lit.

    - Flash point	not applicable

    - Evaporation rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

no data available

    - Vapour pressure	no data available

    - Vapour density	no data available

    - Relative density	3.6 g/cm3 at 25 °C

    - Water solubility	no data available

    - Partition coefficient: n- octanol/water

    - Autoignition temperature

    - Decomposition temperature

no data available no data available no data available

| r) | Viscosity | no data available |
| --- | --- | --- |
| s) | Explosive properties | no data available |
| t) | Oxidizing properties | no data available |

# Other safety information

no data available

# STABILITY AND REACTIVITY

  - Reactivity

no data available

# Chemical stability

no data available

# Possibility of hazardous reactions

Reacts violently with water.

# Conditions to avoid

Exposure to moisture.

# Incompatible materials

Oxidizing agents, Water, acids, Oxygen, Chlorinated solvents, Carbon dioxide (CO2), Halogens, Halogenated hydrocarbon, Alcohols, Sulphur compounds, Hydrogen sulfide gas

# Hazardous decomposition products

Other decomposition products - no data available

# TOXICOLOGICAL INFORMATION

  - Information on toxicological effects Acute toxicity

no data available

Inhalation: Irritating to respiratory system.

# Skin corrosion/irritation

no data available

# Serious eye damage/eye irritation

no data available

# Respiratory or skin sensitization

no data available

# Germ cell mutagenicity

no data available

# Carcinogenicity

This product is or contains a component that is not classifiable as to its carcinogenicity based on its IARC, ACGIH, NTP, or EPA classification.

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

Inhalation - May cause respiratory irritation.

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Potential health effects

Inhalation	May be harmful if inhaled. Causes respiratory tract irritation.

Ingestion	May be harmful if swallowed.

Skin	May be harmful if absorbed through skin. Causes skin irritation.

Eyes	Causes serious eye irritation.

# Signs and Symptoms of Exposure

Stomach/intestinal disorders, Nausea, Vomiting, Drowsiness, Dizziness, Gastrointestinal disturbance, Weakness, Tremors, Seizures.

# Additional Information

RTECS: CQ8370000

# ECOLOGICAL INFORMATION

  - Toxicity

Toxicity to fish	mortality NOEC - Cyprinodon variegatus (sheepshead minnow) - 500 mg/l - 96 h

LC50 - Cyprinodon variegatus (sheepshead minnow) - > 500 mg/l - 96 h

# Persistence and degradability

no data available

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

no data available

# Other adverse effects

no data available

# DISPOSAL CONSIDERATIONS

  - Waste treatment methods Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting

as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional waste disposal service to dispose of this material.

# Contaminated packaging

Dispose of as unused product.

# TRANSPORT INFORMATION

  - UN number

ADR/RID: 1400	IMDG: 1400	IATA: 1400

  - UN proper shipping name ADR/RID:	BARIUM IMDG:	BARIUM

IATA:	Barium

| 14.3 | Transport hazard class(es) ADR/RID: 4.3 | IMDG: 4.3 | IATA: 4.3 |
| --- | --- | --- | --- |
| 14.4 | Packaging group ADR/RID: II | IMDG: II | IATA: II |
| 14.5 | Environmental hazards ADR/RID: no | IMDG Marine pollutant: no | IATA: no |

# 14.6	Special precautions for user

no data available

# REGULATORY INFORMATION

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

no data available

# OTHER INFORMATION

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.