SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifiers

Product name	:	Copper(I) oxide

Product Number	:	CC6944-56

Brand	:	Prime Chemicals

Index-No.	:	029-002-00-X

CAS-No.	:	1317-39-1

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# HAZARDS IDENTIFICATION

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Acute toxicity, Oral (Category 4) Acute aquatic toxicity (Category 1) Chronic aquatic toxicity (Category 1)

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

Harmful if swallowed. Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

# Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Pictogram	 Signal word	Warning

Hazard statement(s)

H302	Harmful if swallowed.

H410	Very toxic to aquatic life with long lasting effects.

Precautionary statement(s)

P273	Avoid release to the environment.

P501	Dispose of contents/ container to an approved waste disposal plant.

Supplemental Hazard	none

Statements

# According to European Directive 67/548/EEC as amended.

Hazard symbol(s)	 R-phrase(s)

R22	Harmful if swallowed.

R50/53	Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

S-phrase(s)

S22	Do not breathe dust.

S60	This material and its container must be disposed of as hazardous waste.

S61	Avoid release to the environment. Refer to special instructions/ Safety data sheets.

  - Other hazards - none

# COMPOSITION/INFORMATION ON INGREDIENTS

  - Substances

Synonyms	:	Cuprous oxide

Formula	:	Cu2O

Molecular Weight	:	143.09 g/mol

| Component | Concentration |
| --- | --- |
| Dicopper oxide |  |
| CAS-No. 1317-39-1 EC-No. 215-270-7 Index-No. 029-002-00-X | - |

# FIRST AID MEASURES

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

Symptoms of systemic copper poisoning may include: capillary damage, headache, cold sweat, weak pulse, and kidney and liver damage, central nervous system excitation followed by depression, jaundice, convulsions, paralysis, and coma. Death may occur from shock or renal failure. Chronic copper poisoning is typified by hepatic cirrhosis, brain damage and demyelination, kidney defects, and copper deposition in the cornea as exemplified by humans with Wilson's disease. It has also been reported that copper poisoning has lead to hemolytic anemia and accelerates arteriosclerosis.

# Indication of any immediate medical attention and special treatment needed

no data available

# FIREFIGHTING MEASURES

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

Copper oxides

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# ACCIDENTAL RELEASE MEASURES

  - Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# HANDLING AND STORAGE

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Air and moisture sensitive.

# Specific end uses

no data available

# EXPOSURE CONTROLS/PERSONAL PROTECTION

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Immersion protection Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: > 480 min

Material tested:Dermatril® (Aldrich Z677272, Size M)

Splash protection Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: > 30 min

Material tested:Dermatril® (Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 873000, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an Industrial Hygienist familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Complete suit protecting against chemicals, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher level protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# PHYSICAL AND CHEMICAL PROPERTIES

  - Information on basic physical and chemical properties

    - Appearance	Form: powder Colour: dark red

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	no data available

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 1,230 °C no data available

    - Flash point	not applicable

    - Evaporation rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

no data available

    - Vapour pressure	no data available

    - Vapour density	no data available

    - Relative density	6 g/mL at 25 °C

    - Water solubility	no data available

    - Partition coefficient: n- octanol/water

    - Autoignition temperature

    - Decomposition temperature

no data available no data available no data available

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

Bulk density	1.4 g/l

# STABILITY AND REACTIVITY

  - Reactivity

no data available

# Chemical stability

no data available

# Possibility of hazardous reactions

no data available

# Conditions to avoid

Air Avoid moisture.

# Incompatible materials

Oxidizing agents

# Hazardous decomposition products

Other decomposition products - no data available

# TOXICOLOGICAL INFORMATION

  - Information on toxicological effects Acute toxicity

LD50 Oral - rat - 470 mg/kg

LC50 Inhalation - rat - 4 h - > 50,000 mg/m3 LD50 Dermal - rat - > 2,000 mg/kg

# Skin corrosion/irritation

Skin - rabbit - No skin irritation

# Serious eye damage/eye irritation

Eyes - rabbit - Mild eye irritation

# Respiratory or skin sensitization

no data available

# Germ cell mutagenicity

no data available

# Carcinogenicity

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

Reproductive toxicity - rat - Inhalation

Effects on Fertility: Male fertility index (e.g., # males impregnating females per # males exposed to fertile nonpregnant females).

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Potential health effects

Inhalation	May be harmful if inhaled. May cause respiratory tract irritation.

Ingestion	Harmful if swallowed.

Skin	May be harmful if absorbed through skin. May cause skin irritation.

Eyes	Causes eye irritation.

# Signs and Symptoms of Exposure

Symptoms of systemic copper poisoning may include: capillary damage, headache, cold sweat, weak pulse, and kidney and liver damage, central nervous system excitation followed by depression, jaundice, convulsions, paralysis, and coma. Death may occur from shock or renal failure. Chronic copper poisoning is typified by hepatic cirrhosis, brain damage and demyelination, kidney defects, and copper deposition in the cornea as exemplified by humans with Wilson's disease. It has also been reported that copper poisoning has lead to hemolytic anemia and accelerates arteriosclerosis.

# Additional Information

RTECS: GL8050000

# ECOLOGICAL INFORMATION

  - Toxicity

Toxicity to fish	LC50 - Cyprinodon variegatus (sheepshead minnow) - > 0.17 mg/l - 96.0 h

Toxicity to daphnia and other aquatic invertebrates

EC50 - Daphnia magna (Water flea) - 0.5 mg/l - 48 h

# Persistence and degradability

no data available

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

no data available

# Other adverse effects

Very toxic to aquatic life.

Avoid release to the environment.

# DISPOSAL CONSIDERATIONS

  - Waste treatment methods Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

# Contaminated packaging

Dispose of as unused product.

# TRANSPORT INFORMATION

  - UN number

ADR/RID: 3077	IMDG: 3077	IATA: 3077

# UN proper shipping name

ADR/RID:	ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Dicopper oxide) IMDG:	ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Dicopper oxide)

IATA:	Environmentally hazardous substance, solid, n.o.s. (Dicopper oxide)

# Transport hazard class(es)

ADR/RID: 9	IMDG: 9	IATA: 9

# Packaging group

ADR/RID: III	IMDG: III	IATA: III

# Environmental hazards

ADR/RID: yes	IMDG Marine pollutant: yes	IATA: yes

# Special precautions for user

Further information

EHS-Mark required (ADR 2.2.9.1.10, IMDG code 2.10.3) for single packagings and combination packagings containing inner packagings with Dangerous Goods > 5L for liquids or > 5kg for solids.

# REGULATORY INFORMATION

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

no data available

# OTHER INFORMATION

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.