SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifiers

Product name	:	1,6-Diaminohexane

Product Number	:	CD2304-48

Brand	:	SLS

Index-No.	:	612-104-00-9

CAS-No.	:	124-09-4

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# HAZARDS IDENTIFICATION

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Acute toxicity, Dermal (Category 4) Acute toxicity, Oral (Category 4)

Specific target organ toxicity - single exposure (Category 3) Skin corrosion (Category 1B)

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

Causes burns. Harmful in contact with skin and if swallowed. Irritating to respiratory system.

# Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Pictogram	 Signal word	Danger

Hazard statement(s)

H302	Harmful if swallowed.

H312	Harmful in contact with skin.

H314	Causes severe skin burns and eye damage.

H335	May cause respiratory irritation.

Precautionary statement(s)

P261	Avoid breathing dust/ fume/ gas/ mist/ vapours/ spray.

P280	Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P310	Immediately call a POISON CENTER or doctor/ physician.

Supplemental Hazard Statements

none

# According to European Directive 67/548/EEC as amended.

|  | R-phrase(s) R21/22 R34 R37 | Harmful in contact with skin and if swallowed. Causes burns. Irritating to respiratory system. |
| --- | --- | --- |
|  | S-phrase(s) S22 | Do not breathe dust. |
|  | S26 S36/37/39 | In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear suitable protective clothing, gloves and eye/face protection. |
|  | S45 | In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible). |
| 2.3 | Other hazards - none |  |

Hazard symbol(s)

# COMPOSITION/INFORMATION ON INGREDIENTS

  - Substances

Synonyms	:Hexamethylenediamine

1,6-Hexanediamine

Formula	:	C6H16N2

Molecular Weight	:	116.20 g/mol

| Component | Concentration |
| --- | --- |
| Hexamethylenediamine |  |
| CAS-No. 124-09-4 EC-No. 204-679-6 Index-No. 612-104-00-9 | - |

# FIRST AID MEASURES

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

burning sensation, Cough, wheezing, laryngitis, Shortness of breath, spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin.

# Indication of any immediate medical attention and special treatment needed

no data available

# FIREFIGHTING MEASURES

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

Carbon oxides, nitrogen oxides (NOx)

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# ACCIDENTAL RELEASE MEASURES

  - Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# HANDLING AND STORAGE

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Hygroscopic. Store under inert gas.

# Specific end uses

no data available

# EXPOSURE CONTROLS/PERSONAL PROTECTION

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment

Eye/face protection

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Immersion protection Material: Nitrile rubber

Minimum layer thickness: 0.4 mm Break through time: > 480 min

Material tested:Camatril® (Aldrich Z677442, Size M)

Splash protection Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: > 30 min

Material tested:Dermatril® (Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 873000, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must  be evaluated by an Industrial Hygienist familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Complete suit protecting against chemicals, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such  as NIOSH (US) or CEN (EU).

# PHYSICAL AND CHEMICAL PROPERTIES

  - Information on basic physical and chemical properties

    - Appearance	Form: solid Colour: colourless

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	12.4 at 100 g/l at 25 °C

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 42 - 45 °C - lit. 204 - 205 °C

    - Flash point	80 °C - closed cup

    - Evaporation rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

Upper explosion limit: 6.3 %(V) Lower explosion limit: 0.7 %(V)

    - Vapour pressure	no data available

    - Vapour density	4.01 - (Air = 1.0)

    - Relative density	0.89 g/cm3 at 25 °C

    - Water solubility	no data available

    - Partition coefficient: n- octanol/water

    - Autoignition temperature

    - Decomposition temperature

log Pow: 0.02

no data available no data available

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

no data available

# STABILITY AND REACTIVITY

  - Reactivity

no data available

# Chemical stability

no data available

# Possibility of hazardous reactions

no data available

# Conditions to avoid

no data available

# Incompatible materials

acids, Acid chlorides, Acid anhydrides, Strong oxidizing agents, Carbon dioxide (CO2)

# Hazardous decomposition products

Other decomposition products - no data available

# TOXICOLOGICAL INFORMATION

  - Information on toxicological effects Acute toxicity

LD50 Oral - rat - 750 mg/kg

LD50 Dermal - rabbit - 1,110 mg/kg

# Skin corrosion/irritation

no data available

# Serious eye damage/eye irritation

no data available

# Respiratory or skin sensitization

no data available

# Germ cell mutagenicity

no data available

# Carcinogenicity

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

May cause respiratory irritation.

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Potential health effects

Inhalation	May be harmful if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract. Causes respiratory tract irritation.

Ingestion	Harmful if swallowed. Causes burns.

Skin	Harmful if absorbed through skin. Causes skin burns.

Eyes	Causes eye burns.

# Signs and Symptoms of Exposure

burning sensation, Cough, wheezing, laryngitis, Shortness of breath, spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin.

# Additional Information

RTECS: MO1180000

# ECOLOGICAL INFORMATION

  - Toxicity

Toxicity to fish	LC50 - Leuciscus idus (Golden orfe) - 62 mg/l - 96 h

Toxicity to daphnia and other aquatic invertebrates

EC50 - Daphnia magna (Water flea) - 23.4 mg/l - 48 h

# Persistence and degradability

Biodegradability	Result: 56 % - Partially biodegradable.

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

no data available

# Other adverse effects

Harmful to aquatic life.

# DISPOSAL CONSIDERATIONS

  - Waste treatment methods Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

# Contaminated packaging

Dispose of as unused product.

| 14. | TRANSPORT INFORMATION |  |
| --- | --- | --- |
| 14.1 | UN number ADR/RID: 2280 | IMDG: 2280 |
| 14.2 | UN proper shipping name |  |

IATA: 2280

ADR/RID:	HEXAMETHYLENEDIAMINE, SOLID IMDG:	HEXAMETHYLENEDIAMINE, SOLID

IATA:	Hexamethylenediamine, solid

# Transport hazard class(es)

ADR/RID: 8	IMDG: 8	IATA: 8

# Packaging group

ADR/RID: III	IMDG: III	IATA: III

# Environmental hazards

ADR/RID: no	IMDG Marine pollutant: no	IATA: no

# Special precautions for user

no data available

# REGULATORY INFORMATION

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

no data available

# OTHER INFORMATION

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be  used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.