Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH)

- Identification of the substance/mixture and of the company/undertaking

  - Product identifier

Trade name/designation	n-Hexane 				              Product No.	CH3404-12 & CH3404-24

Substance name	n-Hexane

CAS No.	110-54-3

INDEX no.	601-037-00-0

REACH registration No.	Not yet communicated down the supply chain. other means of identification

  - Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses	for laboratory use and chemical production.

  - Details of the supplier of the safety data sheet

Supplier (manufacturer/importer/only representative/downstream user/distributor)

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

- Hazards identification

  - Classification of the substance or mixture

    - Classification according to Regulation (EC) No. 1272/2008 [CLP]

| hazard classes and hazard categories | Hazard Statements | classification procedure | remark |
| --- | --- | --- | --- |
| Flammable liquid, category 2 | H225 |  |  |
| Reproductive toxicity, catagory 2 | H361f |  |  |

| Aspiration hazard, category 1 | H304 |  |  |
| --- | --- | --- | --- |
| Specific target organ toxicity (repeated exposure), category 2 | H373 |  |  |
| Skin irritation, category 2 | H315 |  |  |
| Specific target organ toxicity (single exposure), category 3, narcotic effect | H336 |  |  |
| Hazardous to the aquatic environment, chronic, category 2 | H411 |  |  |

    - Classification according to Directive 67/548/EEC or 1999/45/EC

| Hazard symbols: | R-phrases |
| --- | --- |
| F | R11 |
| Xi | R38 |
| Xn | R48/20 |
| Xn | R62 |
| Xn | R65 |
|  | R67 |
| N | R51/53 |

  - Label elements

    - Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Signal word	Danger

Hazard Statements

| H225 | Highly flammable liquid and vapour. |
| --- | --- |
| H361f | Suspected of damaging fertility. |
| H304 | May be fatal if swallowed and enters airways. |
| H373 | May cause damage to organs. |
| H315 | Causes skin irritation. |
| H336 | May cause drowsiness or dizziness. |
| H411 | Toxic to aquatic life with long lasting effects. |

Precautionary statements

| P201 | Obtain special instructions before use. |
| --- | --- |
| P210 | Keep away from heat/sparks/open flames/hot surfaces. No smoking. |
| P243 | Take precautionary measures against static discharge. |
| P281 | Use personal protective equipment as required. |
| P273 | Avoid release to the environment. |
| P301+P331 | IF SWALLOWED: Do NOT induce vomiting. |
| P302+P352 | IF ON SKIN: Wash with plenty of soap and water. |
| P304+P340 | IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. |
| P309+P310 | IF exposed or if you feel unwell: Immediately call a POISON CENTER or doctor/physician. |

2.2.2 Labelling (67/548/EEC or 1999/45/EC)

Hazard symbols: F, Xn, N

R-phrases

| R11 | Highly flammable. |
| --- | --- |
| R38 | Irritating to skin. |
| R48/20 | Harmful: danger of serious damage to health by prolonged exposure through inhalation. |
| R62 | Possible risk of impaired fertility. |
| R65 | Harmful: may cause lung damage if swallowed. |
| R67 | Vapours may cause drowsiness and dizziness. |
| R51/53 | Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |

S-phrases

| S9 | Keep container in a well-ventilated place. |
| --- | --- |
| S16 | Keep away from sources of ignition - No smoking. |
| S29 | Do not empty into drains. |
| S33 | Take precautionary measures against static discharges. |
| S36/37 | Wear suitable protective clothing and gloves. |
| S61 | Avoid release to the environment. Refer to special instructions/safety data sheets. |
| S62 | If swallowed, do not induce vomiting: seek medical advice immediately and show this container or label. |

2.3 Other hazards

SVHC	No

- Composition/ Information on ingredients

Molecular formula	H₃C(CH₂)₄CH₃

Molecular weight (g/mol)	86.18 g/mol

CAS No.	110-54-3

EC No	203-777-6

INDEX no.	601-037-00-0

- First-aid measures

  - General information

IF exposed: Immediately call a POISON CENTER or doctor/ physician. If unconscious place in recovery position and seek medical advice. Never give anything by mouth to an unconscious person or a person with cramps. Change contaminated, saturated clothing. Do not leave affected person unattended.

  - After inhalation

Immediately call a POISON CENTER or doctor/ physician. Remove casualty to fresh air and keep warm and at rest. If breathing is irregular or stopped, administer artificial respiration.

  - In case of skin contact

After contact with skin, wash immediately with plenty of water and soap. Remove contaminated, saturated clothing immediately. In case of skin reactions, consult a physician.

  - After eye contact

In case of contact with eyes flush immediately with plenty of flowing water for 10 to 15 minutes holding eyelids apart and consult an ophthalmologist. Protect uninjured eye. Remove contact lenses, if present and easy to do. Continue rinsing.

  - After ingestion

Immediately call a POISON CENTER or doctor/ physician. Do not induce vomiting. Rinse mouth thoroughly with water. Give nothing to eat or drink.

  - Self-protection of the first aider

First aider: Pay attention to self-protection!

  - Information to physician:

Symptoms	No data available

Hazards	No data available

Treatment	No data available

- Firefighting measures

  - Suitable extinguishing media

Water spray ABC-powder Carbon dioxide (CO2) Nitrogen

  - Extinguishing media which must not be used for safety reasons:

no restriction

  - Special hazards arising from the substance or mixture

In case of fire may be liberated: Carbon dioxide (CO2) Carbon monoxide

  - Advice for firefighters

DO NOT fight fire when fire reaches explosives. In case of fire: Wear self-contained breathing apparatus.

  - Additional information

Do not allow run-off from fire-fighting to enter drains or water courses. Do not inhale explosion and combustion gases. Use caution when applying carbon dioxide in confined spaces. Carbon dioxide can displace oxygen. Use water spray jet to protect personnel and to cool endangered containers.

- Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Avoid generation of dust. Do not breathe dust/fume/gas/mist/vapours/spray. Provide adequate ventilation. Remove all sources of ignition. Use personal protection equipment. The accumulation in lowlying or closen rooms can cause increased danger of fire and explosion. The vapour of the product is heavier than air and may accumulate below ground level, in pits, channels and basements in higher concentration. In case of major fire and large quantities: Remove persons to safety.

  - Environmental precautions

Do not allow to enter into surface water or drains. Make sure spills can be contained, e.g. in sump pallets or kerbed areas. Discharge into the environment must be avoided.

  - Methods and material for containment and cleaning up

Spilled product must never be returned to the original container for recycling. Prevent spread over a wide area (e.g. by containment or oil barriers). Soak up inert absorbent and dispose as waste requiring special attention.

  - Additional information

Clear spills immediately.

- Handling and storage

  - Precautions for safe handling

Avoid: Inhalation. Avoid contact with skin and eyes. Use extractor hood (laboratory). If handled uncovered, arrangements with local exhaust ventilation have to be used. If local exhaust ventilation is not possible or not sufficient, the entire working area must be ventilated by technical means. Keep away from sources of ignition. - No smoking. Usual measures for fire prevention. Take precautionary measures against static discharge. Have fire- extinguishers in readiness before opening containers. Reignition possible over considerable distance.

  - Conditions for safe storage, including any incompatibilities

storage temperature	15-25°C

Keep container tightly closed in a cool, well-ventilated place. Always close containers tightly after the removal of product.

  - Specific end use(s)

No data available

- Exposure controls / Personal protection

  - Control parameters

| Ingredient (Designation) | Limit value type (country of | Limit value | Regulatory information |
| --- | --- | --- | --- |

|  | origin): |  |  |
| --- | --- | --- | --- |
| n-Hexane | TWA (EU) | 20 ppm, 72 mg/m3 | 2006/15/EC |

  - Appropriate engineering controls

Technical measures and the application of suitable work processes have priority over personal protection equipment. If handled uncovered, arrangements with local exhaust ventilation have to be used.

  - Personal protective equipment

Wear suitable protective clothing. When handling with chemical substances, protective clothing with CE-labels including the four control digits must be worn.

    - Eye / face protection

Eye glasses with side protection DIN-/EN-Norms: DIN EN 166

    - Skin protection

When handling with chemical substances, protective gloves must be worn with the CE-label including the four control digits. Recommended glove articles DIN-/EN-Norms: DIN EN 374 In the case of wanting to use the gloves again, clean them before taking off and air them well.

By short-term hand contact

Suitable material:	NBR (Nitrile rubber)

Thickness of the glove material	0,12 mm

Breakthrough time (maximum wearing time)	>480 min

Recommended glove articles	VWR 112-0998

By long-term hand contact

Suitable material:	NBR (Nitrile rubber)

Thickness of the glove material	0,38 mm

Breakthrough time (maximum wearing time)	>480 min

Recommended glove articles	VWR 112-3717 / 112-1381

    - Protective clothing

Wash hands before breaks and after work. Avoid contact with skin and eyes. When using do not eat, drink or smoke. Provide eye shower and label its location conspicuously.

    - Respiratory protection

Respiratory protection necessary at: aerosol or mist formation

Suitable respiratory protection apparatus:	Full-/half-/quarter-face masks (DIN EN 136/140) Recommendation	VWR 111-0206

Suitable material:	A2B2E2K2P3

Recommendation	VWR 111-0059

8.4 Additional information

Wash hands before breaks and after work. Avoid contact with skin and eyes. When using do not eat, drink or smoke.

label its location conspicuously.

- Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance

Physical state	liquid

Colour	colourless

    - Odour	No data available

    - Odour threshold	No data available

# Safety relevant basic data

    - pH	No data available

    - Melting point/freezing point	-94.3°C

    - Initial boiling point and boiling range	69°C (1013 hPa)

    - Flash point	-22°C

    - Evaporation rate	No data available

    - Flammability (solid, gas)	Highly flammable liquid and vapour.

    - Upper/lower flammability or explosive limits

Lower explosion limit (Vol-%)	1.1

Upper explosion limit (Vol-%)	8.1

    - Vapour pressure	160 hPa (20°C)

    - Vapour density	2.79 (20°C)

    - Relative density	0.659 g/cm³ (20°C)

    - Solubility(ies)

Water solubility (g/l)	0,0095 g/l (20°C) at °C:	20

Soluble (g/l) in	No data available

    - Partition coefficient: n-octanol/water	3.94 (20°C)

    - Auto-ignition temperature	240°C

    - Decomposition temperature	No data available

    - Viscosity

Kinematic viscosity	No data available

Dynamic viscosity	0.326 mPa*s (20°C)

    - Explosive properties	not applicable

    - Oxidising properties	not applicable

  - Other information

Bulk density	No data available

refraction index	1.375 (589 nm, 20°C)

dissociation constant	No data available

Surface tension	No data available

Henry constant	No data available

- Stability and reactivity

  - Reactivity

Vapours can form explosive mixtures with air.

  - Chemical stability

The product is chemically stable under standard ambient conditions (room temperature).

  - Possibility of hazardous reactions

Formation of potentially explosive mixtures with: Oxidising agent, strong

  - Conditions to avoid

This material is combustible and can be ignited by heat, sparks, flames, or other sources of ignition (e.g. static electricity, pilot lights, or mechanical/electrical equipment).

  - Incompatible materials

Rubber articles Plastic articles

  - Hazardous decomposition products

No data available

  - Additional information

No data available

- Toxicological information

  - Information on toxicological effects

# Acute effects

Acute oral toxicity

Effective dose	LD50: 25000 mg/kg

species:	rat

Exposure time remark

source	RTECS

Acute dermal toxicity

Effective dose	LD50: Min. 2000 mg/kg

species:	rabbit

Exposure time remark

source	Merck KGaA

Acute inhalation toxicity

Effective dose	No data available

species:	No data available

Exposure time remark  source

# Irritant and corrosive effects

Primary irritation to the skin Exposure time

species:   Result

Irritation to eyes Exposure time species:   Result

Irritation to respiratory tract Exposure time

species: Result

# Sensitisation

In case of skin contact	not sensitising.

After inhalation	not sensitising.

# Specific target organ toxicity (single exposure)

May cause drowsiness or dizziness.

# Specific target organ toxicity (repeated exposure)

May cause damage to organs.

# CMR effects (carcinogenicity, mutagenicity and toxicity for reproduction)

Carcinogenicity

No indication of human carcinogenicity.

# Germ cell mutagenicity/Genotoxicity

No indications of human germ cell mutagenicity exist.

Reproductive toxicity Suspected of damaging fertility. Aspiration hazard

May be fatal if swallowed and enters airways.

  - Other adverse effects

No data available

  - Additional information

No data available

- Ecological information

  - Ecotoxicity

# Acute (short-term) fish toxicity

LC50:	No data available

EC50

species: Exposure time

# Chronic (long-term) fish toxicity

LC50:	No data available

EC50

species: Exposure time

# Acute (short-term) daphnia toxicity

LC50:	No data available

EC50

species: Exposure time

# Chronic (long-term) daphnia toxicity

LC50:	No data available

EC50

species: Exposure time

# Acute (short-term) algae toxicity

LC50:	No data available

EC50

species: Exposure time

# Chronic (long-term) algae toxicity

LC50:	No data available

EC50

species: Exposure time

  - Persistence and degradability

No data available

  - Bioaccumulative potential

(o) Partition coefficient: n-octanol/water	3.94 (20°C)

  - Mobility in soil

No data available

  - Results of PBT assessment

No data available

  - Other adverse effects

No data available

- Disposal considerations

  - Waste treatment methods

# Appropriate disposal / Product

Dispose according to legislation. Consult the appropriate local waste disposal expert about waste disposal. Send to a hazardous waste incinerator facility under observation of official regulations.

Waste code product	16 05 08 (discarded organic chemicals consisting of or containing dangerous substances)

# Appropriate disposal / Package

  - Additional information

No data available

- Transport information

  - Land transport (ADR/RID)

UN-No.	1208

Proper Shipping Name	HEXANES

Class(es)	3

Classification code:	F1

Packing group	II

Hazard label(s)	3

  - Sea transport (IMDG)

UN-No.	1208

Proper Shipping Name	HEXANES

Class(es)	3

Classification code:	F1

Packing group	II

Marine pollutant Segregation group

  - Air transport (ICAO-TI / IATA-DGR)

UN-No.	1208

Proper Shipping Name	HEXANES

Class(es)	3

Classification code:	F1

Packing group	II

  - Additional information

No data available

- Regulatory information

  - Safety, health and environmental regulations/legislation specific for the substance or mixture

Water hazard class (WGK)	2

  - Chemical Safety Assessment

No data available

- Other information

  - Relevant R-, H- and EUH-phrases (Number and full text)

| R11 | Highly flammable. |
| --- | --- |
| R38 | Irritating to skin. |
| R48/20 | Harmful: danger of serious damage to health by prolonged exposure through inhalation. |
| R62 | Possible risk of impaired fertility. |
| R65 | Harmful: may cause lung damage if swallowed. |
| R67 | Vapours may cause drowsiness and dizziness. |
| R51/53 | Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |

| H225 | Highly flammable liquid and vapour. |
| --- | --- |
| H361f | Suspected of damaging fertility. |
| H304 | May be fatal if swallowed and enters airways. |
| H373 | May cause damage to organs. |
| H315 | Causes skin irritation. |
| H336 | May cause drowsiness or dizziness. |
| H411 | Toxic to aquatic life with long lasting effects. |

  - Additional information

Indication of changes general update

The above information describes exclusively the safety requirements of the product and is based on our present-day knowledge. The information is intended to give you advice about the safe handling of the product named in this safety data sheet, for storage, processing, transport and disposal. The information cannot be transferred to other products.  In the case of mixing the product with other products or in the case of processing, the information on this safety data sheet is not necessarily valid for the new made-up material.