SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	:	Iron Powder

Product Number	:	CI5950-54

Brand	:	Prime Chemicals

REACH No.	:	01-2119462838-24-XXXX

CAS-No.	:	7439-89-6

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

Not a hazardous substance or mixture according to Regulation (EC) No. 1272/2008. This substance is not classified as dangerous according to Directive 67/548/EEC.

# Label elements

The product does not need to be labelled in accordance with EC directives or respective national laws.

  - Other hazards - none

# SECTION 3: Composition/information on ingredients

| 3.1 | Substances |  |
| --- | --- | --- |
|  | Formula : | Fe |
|  | CAS-No. : | 7439-89-6 |
|  | EC-No. : | 231-096-4 |
|  | Registration number : | 01-2119462838-24-XXXX |

No components need to be disclosed according to the applicable regulations.

# SECTION 4: First aid measures

  - Description of first aid measures If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration.

# In case of skin contact

Wash off with soap and plenty of water.

# In case of eye contact

Flush eyes with water as a precaution.

# If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

no data available

# SECTION 5: Firefighting measures

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

Iron oxides

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Avoid dust formation. Avoid breathing vapours, mist or gas. For personal protection see section 8.

# Environmental precautions

No special environmental precautions required.

# Methods and materials for containment and cleaning up

Sweep up and shovel. Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

Provide appropriate exhaust ventilation at places where dust is formed. For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

# Specific end use(s)

A part from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

General industrial hygiene practice.

# Personal protective equipment

Eye/face protection

Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must  be evaluated by an industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Choose body protection in relation to its type, to the concentration and amount of dangerous substances, and to the specific work-place., The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Respiratory protection is not required. Where protection from nuisance levels of dusts are desired, use type N95 (US) or type P1 (EN 143) dust masks. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# Control of environmental exposure

No special environmental precautions required.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: powder Colour: light grey

    - Odour	odourless

    - Odour Threshold	no data available

    - pH	not applicable

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 1,538 °C at 1,023 hPa 2,861 °C at 1,013 hPa

    - Flash point	not applicable

    - Evapouration rate	no data available

    - Flammability (solid, gas)  not auto-flammable

    - Upper/lower flammability or explosive limits

no data available

    - Vapour pressure	not applicable

    - Vapour density	no data available

    - Relative density	no data available

    - Water solubility	insoluble

    - Partition coefficient: n- octanol/water

    - Auto-ignition temperature

    - Decomposition temperature

not applicable

no data available no data available

    - Viscosity	no data available

    - Explosive properties	Not explosive

    - Oxidizing properties	The substance or mixture is not classified as oxidizing.

| 9.2 | Other safety information |  |
| --- | --- | --- |
|  | Dust explosion class Bulk density | St1 0.002 - 0.003 kg/m3 |

# SECTION 10: Stability and reactivity

  - Reactivity

no data available

# Chemical stability

Stable under recommended storage conditions.

# Possibility of hazardous reactions

no data available

# Conditions to avoid

no data available

# Incompatible materials

Strong oxidizing agents, Strong acids

# Hazardous decomposition products

Other decomposition products - no data available In the event of fire: see section 5

# SECTION 11: Toxicological information

11.1	Information on toxicological effects Acute toxicity

LD50 Oral - rat - 7,500 mg/kg

# Skin corrosion/irritation

No skin irritation

# Serious eye damage/eye irritation

Eyes - rabbit

Result: No eye irritation (OECD Test Guideline 405)

# Respiratory or skin sensitisation

Did not cause sensitisation on laboratory animals.

# Germ cell mutagenicity

S. typhimurium

Result: Not mutagenic in Ames Test.

# Carcinogenicity

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

Did not show teratogenic effects in animal experiments. Animal testing did not show any effects on fertility.

# Specific target organ toxicity - single exposure

The substance or mixture is not classified as specific target organ toxicant, single exposure.

# Specific target organ toxicity - repeated exposure

The substance or mixture is not classified as specific target organ toxicant, repeated exposure.

# Aspiration hazard

no data available

# Additional Information

RTECS: Not available

# SECTION 12: Ecological information

  - Toxicity

Toxicity to fish	static test - Morone saxatilis - 13.6 mg/l - 96 h

# Persistence and degradability

not applicable

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

# Other adverse effects

no data available

# SECTION 13: Disposal considerations

13.1	Waste treatment methods Product

Offer surplus and non-recyclable solutions to a licensed disposal company.

# Contaminated packaging

Dispose of as unused product.

# SECTION 14: Transport information

| 14.1 | UN number |  |
| --- | --- | --- |
| 14.2 | ADR/RID: - UN proper shipping name | IMDG: - |

IATA: -

ADR/RID: Not dangerous goods IMDG: Not dangerous goods IATA:          Not dangerous goods

# 14.3	Transport hazard class(es)

ADR/RID: -                                      IMDG: -                                           IATA: -

| 14.4 | Packaging group ADR/RID: - | IMDG: - | IATA: - |
| --- | --- | --- | --- |
| 14.5 | Environmental hazards ADR/RID: no | IMDG Marine pollutant: no | IATA: no |
| 14.6 | Special precautions for user no data available |  |  |

# SECTION 15: Regulatory information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

# SECTION 16: Other information

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.