SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifiers

Product name	:	Benzoyl chloride

| Product Number : | CB2854-16 |
| --- | --- |
| Brand : | Prime Chemiclas CheChemicals |
| Index-No. : | 607-012-00-0 |
| CAS-No. : | 98-88-4 |

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# HAZARDS IDENTIFICATION

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Acute toxicity, Oral (Category 4) Acute toxicity, Inhalation (Category 4) Acute toxicity, Dermal (Category 4) Skin corrosion (Category 1B)

Skin sensitization (Category 1)

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

Causes burns. Harmful by inhalation, in contact with skin and if swallowed. May cause sensitization by skin contact.

# Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Pictogram	 Signal word	Danger

Hazard statement(s)

H302 + H312 + H332	Harmful if swallowed, in contact with skin or if inhaled H314	Causes severe skin burns and eye damage.

H317	May cause an allergic skin reaction.

Precautionary statement(s)

P280	Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P310	Immediately call a POISON CENTER or doctor/ physician.

Supplemental Hazard Statements

none

# According to European Directive 67/548/EEC as amended.

Hazard symbol(s)	 R-phrase(s)

R20/21/22	Harmful by inhalation, in contact with skin and if swallowed.

R34	Causes burns.

R43	May cause sensitization by skin contact.

S-phrase(s)

S26	In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36/37/39	Wear suitable protective clothing, gloves and eye/face protection.

S45	In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

# Other hazards

Lachrymator. Lachrymator.

# COMPOSITION/INFORMATION ON INGREDIENTS

  - Substances

Formula	:	C7H5ClO

Molecular Weight	:	140.57 g/mol

| Component | Concentration |
| --- | --- |
| Benzoyl chloride |  |
| CAS-No. 98-88-4 EC-No. 202-710-8 Index-No. 607-012-00-0 | - |

# FIRST AID MEASURES

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, burning sensation, Cough, wheezing, laryngitis, Shortness of breath, Headache, Nausea, Vomiting, Lachrymation, Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

# Indication of any immediate medical attention and special treatment needed

no data available

# FIREFIGHTING MEASURES

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

Carbon oxides, Hydrogen chloride gas

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

Water hydrolyzes material liberating acidic gas which in contact with metal surfaces can generate flammable and/or explosive hydrogen gas.Use water spray to cool unopened containers.

# ACCIDENTAL RELEASE MEASURES

  - Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep in suitable, closed containers for disposal.

# Reference to other sections

For disposal see section 13.

# HANDLING AND STORAGE

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist.

Keep away from sources of ignition - No smoking.Take measures to prevent the build up of electrostatic charge.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Store under inert gas. Moisture sensitive.

# Specific end uses

no data available

# EXPOSURE CONTROLS/PERSONAL PROTECTION

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment Eye/face protection

Tightly fitting safety goggles. Faceshield (8-inch minimum). Use equipment for eye protection

tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Immersion protection Material: Fluorinated rubber

Minimum layer thickness: 0.7 mm Break through time: > 480 min

Material tested:Vitoject® (Aldrich Z677698, Size M)

Splash protection

Material: Fluorinated rubber Minimum layer thickness: 0.7 mm Break through time: > 30 min

Material tested:Vitoject® (Aldrich Z677698, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 873000, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an Industrial Hygienist familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Complete suit protecting against chemicals, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# PHYSICAL AND CHEMICAL PROPERTIES

  - Information on basic physical and chemical properties

    - Appearance	Form: clear, liquid Colour: colourless

    - Odour	pungent

    - Odour Threshold	no data available

    - pH	2 at 1 g/l

    - Melting point/freezing point

Melting point/range: -1 °C

    - Initial boiling point and	198 °C

boiling range

    - Flash point	72 °C - closed cup

    - Evaporation rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

Upper explosion limit: 27 %(V) Lower explosion limit: 2.5 %(V)

    - Vapour pressure	1 hPa at 32 °C

    - Vapour density	4.85 - (Air = 1.0)

    - Relative density	1.211 g/mL at 25 °C

    - Water solubility	2 g/l

    - Partition coefficient: n- octanol/water

    - Autoignition temperature

    - Decomposition temperature

no data available

600 °C at 1,013 hPaAuto-flammability no data available

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

no data available

# STABILITY AND REACTIVITY

  - Reactivity

no data available

# Chemical stability

no data available

# Possibility of hazardous reactions

no data available

  - Conditions to avoid Exposure to moisture. Heat, flames and sparks.

# Incompatible materials

Strong oxidizing agents, Strong bases, Alcohols

# Hazardous decomposition products

Other decomposition products - no data available

# TOXICOLOGICAL INFORMATION

  - Information on toxicological effects Acute toxicity

LD50 Oral - rat - female - 1,900 mg/kg LD50 Oral - rat - male - 3,619 mg/kg LC50 Inhalation - rat - 2 h - 1,870 mg/m3 LD50 Dermal - rabbit - 790 mg/kg

# Skin corrosion/irritation

Skin - rabbit - Severe skin irritation - 24 h

# Serious eye damage/eye irritation

Eyes - rabbit - Severe eye irritation

# Respiratory or skin sensitization

Maximisation Test - guinea pig - OECD Test Guideline 406 - May cause sensitization by skin contact.

# Germ cell mutagenicity

no data available

# Carcinogenicity

This product is or contains a component that is not classifiable as to its carcinogenicity based on its IARC, ACGIH, NTP, or EPA classification.

IARC:	2A - Group 2A: Probably carcinogenic to humans (Benzoyl chloride)

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Potential health effects

Inhalation	Harmful if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract.

Ingestion	Harmful if swallowed. Causes burns.

Skin	Harmful if absorbed through skin. Causes skin burns.

Eyes	Causes eye burns.

# Signs and Symptoms of Exposure

spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, burning sensation, Cough, wheezing, laryngitis, Shortness of breath, Headache, Nausea, Vomiting, Lachrymation, Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

# Additional Information

RTECS: DM6600000

# ECOLOGICAL INFORMATION

  - Toxicity

Toxicity to fish	LC50 - Pimephales promelas (fathead minnow) - 34.1 mg/l - 96 h LC0 - Danio rerio (zebra fish) - 7.5 mg/l - 96 h

# Persistence and degradability

Biodegradability	aerobic - Exposure time 20 d

Result: 95 % - Readily biodegradable. Method: Closed Bottle test

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

no data available

  - Other adverse effects Harmful to aquatic life. no data available

# DISPOSAL CONSIDERATIONS

  - Waste treatment methods Product

This combustible material may be burned in a chemical incinerator equipped with an afterburner and

scrubber. Offer surplus and non-recyclable solutions to a licensed disposal company.

# Contaminated packaging

Dispose of as unused product.

| 14. | TRANSPORT INFORMATION |  |
| --- | --- | --- |
| 14.1 | UN number ADR/RID: 1736 | IMDG: 1736 |
| 14.2 | UN proper shipping name |  |

IATA: 1736

ADR/RID:	BENZOYL CHLORIDE IMDG:	BENZOYL CHLORIDE

IATA:	Benzoyl chloride

# Transport hazard class(es)

ADR/RID: 8	IMDG: 8	IATA: 8

# Packaging group

ADR/RID: II	IMDG: II	IATA: II

# Environmental hazards

ADR/RID: no	IMDG Marine pollutant: no	IATA: no

# Special precautions for user

no data available

# REGULATORY INFORMATION

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

no data available

# OTHER INFORMATION

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.