SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	:	Aluminum Powder Fine

Product Number	:	CA3520-54

Brand	:	PRIME CHEMICALS

Index-No.	:	013-002-00-1

REACH No.	:	01-2119529243-45-XXXX

CAS-No.	:	7429-90-5

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Flammable solids (Category 1), H228

For the full text of the H-Statements mentioned in this Section, see Section 16.

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

F	Highly flammable	R11

For the full text of the R-phrases mentioned in this Section, see Section 16.

# Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram	 Signal word	Danger

Hazard statement(s)

H228	Flammable solid.

Precautionary statement(s)

P210	Keep away from heat/sparks/open flames/hot surfaces. - No smoking.

P240	Ground/bond container and receiving equipment.

P241	Use explosion-proof electrical/ ventilating/ lighting/ equipment.

P280	Wear protective gloves/ protective clothing/ eye protection/ face protection.

P370 + P378	In case of fire: Use sand for extinction.

Supplemental Hazard Statements

  - Other hazards - none

none

# SECTION 3: Composition/information on ingredients

| 3.1 | Substances |  |
| --- | --- | --- |
|  | Formula : | Al |
|  | Molecular Weight : | 26.98 g/mol |
|  | CAS-No. : | 7429-90-5 |
|  | EC-No. : | 231-072-3 |
|  | Index-No. : | 013-002-00-1 |
|  | Registration number : | 01-2119529243-45-XXXX |

No components need to be disclosed according to the applicable regulations.

For the full text of the H-Statements and R-Phrases mentioned in this Section, see Section 16

# SECTION 4: First aid measures

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

# In case of eye contact

Flush eyes with water as a precaution.

# If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

no data available

# SECTION 5: Firefighting measures

  - Extinguishing media  Suitable extinguishing media

Special powder against metal fire Dry sand Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

# Unsuitable extinguishing media

Water Carbon dioxide (CO2) ABC powder

# Special hazards arising from the substance or mixture

Aluminum oxide

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

no data available

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas.

For personal protection see section 8.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

# Methods and materials for containment and cleaning up

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Do not flush with water. Keep in suitable, closed containers for disposal. Contain spillage, pick up with an electrically protected vacuum cleaner or by wet-brushing and transfer to a container for disposal according to local regulations (see section 13).

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.Keep away from sources of ignition

- No smoking.Take measures to prevent the build up of electrostatic charge. For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Store in original container. Do not store near combustible materials. Keep in a cool place away from acids. Keep in a cool place away from bases. Keep in a cool place away from oxidizing agents.

Handle and store under inert gas.

# Specific end use(s)

A part from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

Components with workplace control parameters

| Component | CAS-No. | Value | Control parameters | Basis |
| --- | --- | --- | --- | --- |
|  | 7429-90-5 | TWA | 10 mg/m3 | UK. EH40 WEL - Workplace Exposure Limits |
|  | Remarks | For the purposes of these limits, respirable dust and inhalable dust are those fractions of airborne dust which will be collected when sampling is undertaken in accordance with the methods described in MDHS14/3 General methods for sampling and gravimetric analysis of respirable and inhalable dust The COSHH definition of a substance hazardous to health includes dust of any kind when present at a concentration in air equal to or greater than 10 mg.m-3 8-hour TWA of inhalable dust or 4 mg.m-3 8-hour TWA of respirable dust. This means that any dust will be subject to COSHH if people are exposed above these levels. Some dusts have been assigned specific WELs and exposure to these must comply with the appropriate limit. Most industrial dusts contain particles of a wide range of sizes. The behaviour, deposition and fate of any particular particle after entry into the human respiratory system and the body response that it elicits, depend on the nature and size of the particle. HSE distinguishes two size fractions for limit-setting purposes termed |  |  |

|  |  | 'inhalable' and 'respirable'. Inhalable dust approximates to the fraction of airborne material that enters the nose and mouth during breathing and is therefore available for deposition in the respiratory tract. Respirable dust approximates to the fraction that penetrates to the gas exchange region of the lung. Fuller definitions and explanatory material are given in MDHS14/3. Where dusts contain components that have their own assigned WEL, all the relevant limits should be complied with. Where no specific short-term exposure limit is listed, a figure three times the long-term exposure should be used |  |  |
| --- | --- | --- | --- | --- |
|  |  | TWA | 4 mg/m3 | UK. EH40 WEL - Workplace Exposure Limits |
|  |  | For the purposes of these limits, respirable dust and inhalable dust are those fractions of airborne dust which will be collected when sampling is undertaken in accordance with the methods described in MDHS14/3 General methods for sampling and gravimetric analysis of respirable and inhalable dust The COSHH definition of a substance hazardous to health includes dust of any kind when present at a concentration in air equal to or greater than 10 mg.m-3 8-hour TWA of inhalable dust or 4 mg.m-3 8-hour TWA of respirable dust. This means that any dust will be subject to COSHH if people are exposed above these levels. Some dusts have been assigned specific WELs and exposure to these must comply with the appropriate limit. Most industrial dusts contain particles of a wide range of sizes. The behaviour, deposition and fate of any particular particle after entry into the human respiratory system and the body response that it elicits, depend on the nature and size of the particle. HSE distinguishes two size fractions for limit-setting purposes termed 'inhalable' and 'respirable'. Inhalable dust approximates to the fraction of airborne material that enters the nose and mouth during breathing and is therefore available for deposition in the respiratory tract. Respirable dust approximates to the fraction that penetrates to the gas exchange region of the lung. Fuller definitions and explanatory material are given in MDHS14/3. Where dusts contain components that have their own assigned WEL, all the relevant limits should be complied with. Where no specific short-term exposure limit is listed, a figure three times the long-term exposure should be used |  |  |

  - Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

# Personal protective equipment Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Protective gloves against thermal risks

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact Material: Nitrile rubber

Minimum layer thickness: 0.11 mm Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Flame retardant antistatic protective clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such  as NIOSH (US) or CEN (EU).

# Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: powder Colour: silver

    - Odour	odourless

    - Odour Threshold	no data available

    - pH	no data available

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 660 °C 2,467 °C

    - Flash point	not applicable

    - Evapouration rate	no data available

    - Flammability (solid, gas)  The substance or mixture is a flammable solid with the category 1.

    - Upper/lower flammability or explosive limits

no data available

    - Vapour pressure	no data available

    - Vapour density	no data available

    - Relative density	2.7 g/mL at 25 °C

    - Water solubility	insoluble

    - Partition coefficient: n-	no data available

octanol/water

    - Auto-ignition temperature

    - Decomposition temperature

not auto-flammable not applicable

    - Viscosity	no data available

    - Explosive properties	Risk of dust explosion.

    - Oxidizing properties	no data available

# Other safety information

no data available

# SECTION 10: Stability and reactivity

  - Reactivity

no data available

# Chemical stability

Stable under recommended storage conditions.

# Possibility of hazardous reactions

Risk of dust explosion.Reacts with water to generate Hydrogen gas.Reacts with the following substances:, Acids, Bases, Oxidizing agents, Halogens

# Conditions to avoid

Heat, flames and sparks. Extremes of temperature and direct sunlight. Humid air water

# Incompatible materials

Acids, Bases, Halogens, Oxidizing agents

# Hazardous decomposition products

Other decomposition products - no data available In the event of fire: see section 5

# SECTION 11: Toxicological information

11.1	Information on toxicological effects Acute toxicity

LD50 Oral - rat - > 2,000 mg/kg

LC50 Inhalation - rat - 4 h - > 888 mg/l

# Skin corrosion/irritation

no data available

# Serious eye damage/eye irritation

no data available

# Respiratory or skin sensitisation

no data available

# Germ cell mutagenicity

no data available

# Carcinogenicity

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Additional Information

RTECS: BD0330000

Cough, weight loss, anemia, Weakness, Incoordination.

# SECTION 12: Ecological information

  - Toxicity

no data available

# Persistence and degradability

no data available

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

# Other adverse effects

no data available

# SECTION 13: Disposal considerations

13.1	Waste treatment methods Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting

as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

# Contaminated packaging

Dispose of as unused product.

# SECTION 14: Transport information

| 14.1 | UN number |  |
| --- | --- | --- |
| 14.2 | ADR/RID: 1309 UN proper shipping name | IMDG: 1309 |

IATA: 1309

ADR/RID:	ALUMINIUM POWDER, COATED IMDG:	ALUMINIUM POWDER, COATED

IATA:	Aluminium powder, coated

| 14.3 | Transport hazard class(es) ADR/RID: 4.1 | IMDG: 4.1 | IATA: 4.1 |
| --- | --- | --- | --- |
| 14.4 | Packaging group ADR/RID: II | IMDG: II | IATA: II |
| 14.5 | Environmental hazards ADR/RID: no | IMDG Marine pollutant: no | IATA: no |
| 14.6 | Special precautions for user no data available |  |  |

# SECTION 15: Regulatory information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

# SECTION 16: Other information

Full text of H-Statements referred to under sections 2 and 3.

H228	Flammable solid.

# Full text of R-phrases referred to under sections 2 and 3

R11	Highly flammable.

# Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be  used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.