SAFETY DATA SHEET

according to Regulation (EC) No. 1907/2006 (Reach)

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	:	Ethylenediamine

Product Number	:	CD1854-16

Brand	:	Prime Chemicals

Index-No.	:	612-006-00-6

REACH No.	:	A registration number is not available for this substance as the substance

or its uses are exempted from registration, the annual tonnage does not require a registration or the registration is envisaged for a later registration deadline.

CAS-No.	:	107-15-3

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	:	Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

PRIME CHEMICALS

Company	:	Prime Chemicals

Northfield Road

Rotherham

S60 1RR

Telephone	:	+44 (0) 1709 377881

Fax	:	+44 (0) 1709 369264

# Emergency telephone number

Emergency Phone #	:	+44 (0) 1709 377881 (Only manned during normal working hours)

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Flammable liquids (Category 3), H226  Acute toxicity, Oral (Category 4), H302 Acute toxicity, Inhalation (Category 4), H332 Acute toxicity, Dermal (Category 3), H311 Skin corrosion (Category 1B), H314

Respiratory sensitisation (Category 1), H334 Skin sensitisation (Category 1), H317 Chronic aquatic toxicity (Category 3), H412

For the full text of the H-Statements mentioned in this Section, see Section 16.

# Classification according to EU Directives 67/548/EEC or 1999/45/EC

R10

| C | Corrosive | R34 |
| --- | --- | --- |
| Xn | Harmful | R21/22 |
|  |  | R42/43 |

For the full text of the R-phrases mentioned in this Section, see Section 16.

# Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram

Signal word	Danger

Hazard statement(s)

H226	Flammable liquid and vapour.

H302 + H332	Harmful if swallowed or if inhaled

H311	Toxic in contact with skin.

H314	Causes severe skin burns and eye damage.

H317	May cause an allergic skin reaction.

H334	May cause allergy or asthma symptoms or breathing difficulties if inhaled.

H412	Harmful to aquatic life with long lasting effects.

Precautionary statement(s)

P261	Avoid breathing vapours.

P273	Avoid release to the environment.

P280	Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P310	Immediately call a POISON CENTER or doctor/ physician.

Supplemental Hazard Statements

# Other hazards

Rapidly absorbed through skin. Lachrymator.

none

# SECTION 3: Composition/information on ingredients

| 3.1 | Substances Synonyms : | 1,2-Diaminoethane |
| --- | --- | --- |
|  | Formula : Molecular Weight : CAS-No. : | C2H8N2 60.10 g/mol 107-15-3 |
|  | EC-No. : | 203-468-6 |
|  | Index-No. : | 612-006-00-6 |

Hazardous ingredients according to Regulation (EC) No 1272/2008

| Component | Classification | Concentration |
| --- | --- | --- |
| Ethylenediamine |  |  |
| CAS-No. 107-15-3 EC-No. 203-468-6 Index-No. 612-006-00-6 | Flam. Liq. 3; Acute Tox. 4; Acute Tox. 3; Skin Corr. 1B; Resp. Sens. 1; Skin Sens. 1; Aquatic Chronic 3; H226, H311, H302 + H332, H314, H317, H334, H412 | <= 100 % |

Hazardous ingredients according to Directive 1999/45/EC

| Component | Classification | Concentration |
| --- | --- | --- |
| Ethylenediamine |  |  |
| CAS-No. 107-15-3 EC-No. 203-468-6 Index-No. 612-006-00-6 | C, R10 - R21/22 - R34 - R42/43 | <= 100 % |

For the full text of the H-Statements and R-Phrases mentioned in this Section, see Section 16

# SECTION 4: First aid measures

  - Description of first aid measures General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

# If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

# In case of skin contact

Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

# In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

# If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

no data available

# SECTION 5: Firefighting measures

  - Extinguishing media  Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# Special hazards arising from the substance or mixture

Carbon oxides, nitrogen oxides (NOx)

Flash back possible over considerable distance., Container explosion may occur under fire conditions., Vapours may form explosive mixture with air.

# Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

# Further information

Use water spray to cool unopened containers.

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Wear respiratory protection. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas.

For personal protection see section 8.

# Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# Methods and materials for containment and cleaning up

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13).

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist.

Keep away from sources of ignition - No smoking.Take measures to prevent the build up of electrostatic charge.

For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Air and moisture sensitive. Handle and store under inert gas.

# Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

# Exposure controls

Appropriate engineering controls

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

# Personal protective equipment Eye/face protection

Tightly fitting safety goggles. Faceshield (8-inch minimum). Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

# Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Full contact

Material: butyl-rubber

Minimum layer thickness: 0.3 mm Break through time: 480 min

Material tested:Butoject® (KCL 897 / Aldrich Z677647, Size M)

Splash contact

Material: Nature latex/chloroprene Minimum layer thickness: 0.6 mm Break through time: 72 min

Material tested:Lapren® (KCL 706 / Aldrich Z677558, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method: EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be construed as offering an approval for any specific use scenario.

# Body Protection

Complete suit protecting against chemicals, Flame retardant antistatic protective clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

# Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

# Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: liquid

    - Odour	no data available

    - Odour Threshold	no data available

    - pH	12.2 at 110 g/l at 20 °C

    - Melting point/freezing point

    - Initial boiling point and boiling range

Melting point/range: 8.5 °C 118 °C

    - Flash point	38 °C - closed cup

    - Evapouration rate	no data available

    - Flammability (solid, gas)  no data available

    - Upper/lower flammability or explosive limits

Upper explosion limit: 16 %(V) Lower explosion limit: 2.7 %(V)

    - Vapour pressure	13 hPa at 20 °C

    - Vapour density	2.07 - (Air = 1.0)

    - Relative density	0.899 g/mL at 25 °C

    - Water solubility	soluble

    - Partition coefficient: n- octanol/water

    - Auto-ignition temperature

    - Decomposition temperature

log Pow: -2.04 no data available no data available

    - Viscosity	no data available

    - Explosive properties	no data available

    - Oxidizing properties	no data available

# Other safety information

Relative vapour density	2.07 - (Air = 1.0)

# SECTION 10: Stability and reactivity

  - Reactivity

no data available

# Chemical stability

Absorbs carbon dioxide (CO2) from air.

Stable under recommended storage conditions.

# Possibility of hazardous reactions

no data available

# Conditions to avoid

Air Exposure to moisture. Heat, flames and sparks.

# Incompatible materials

Oxidizing agents, Phosphorus halides, Aldehydes, Organic halides

# Hazardous decomposition products

Other decomposition products - no data available In the event of fire: see section 5

# SECTION 11: Toxicological information

11.1	Information on toxicological effects Acute toxicity

LD50 Oral - rat - 1,200 mg/kg Remarks: Behavioral:Ataxia.

LC50 Inhalation - rat - 4 h - 14.7 mg/l LD50 Dermal Dermal - rabbit - 560 mg/kg

# Skin corrosion/irritation

Skin - rabbit

Result: Causes burns.

# Serious eye damage/eye irritation

Eyes - rabbit Result: Corrosive

Respiratory or skin sensitisation Maximisation Test - guinea pig Result: Causes sensitisation.

May cause allergic respiratory and skin reactions

# Germ cell mutagenicity

no data available

# Carcinogenicity

Carcinogenicity - This product is or contains a component that is not classifiable as to its carcinogenicity based on its IARC, ACGIH, NTP, or EPA classification.

IARC:	No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

no data available

# Specific target organ toxicity - single exposure

no data available

# Specific target organ toxicity - repeated exposure

no data available

# Aspiration hazard

no data available

# Additional Information

RTECS: KH8575000

Vomiting, Diarrhoea, Abdominal pain

Liver - Irregularities - Based on Human Evidence

# SECTION 12: Ecological information

  - Toxicity

Toxicity to fish	LC50 - Pimephales promelas (fathead minnow) - 115.7 mg/l - 96 h

Toxicity to daphnia and other aquatic invertebrates

EC50 - Daphnia magna (Water flea) - 3 mg/l - 48 h

Toxicity to algae	EC50 - Pseudokirchneriella subcapitata (green algae) - 151 mg/l - 96 h

# Persistence and degradability

Biodegradability	Biotic/Aerobic - Exposure time 28 d

Result: 94 % - Readily biodegradable.

# Bioaccumulative potential

no data available

# Mobility in soil

no data available

# Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

# Other adverse effects

Toxic to aquatic life.

Avoid release to the environment.

# SECTION 13: Disposal considerations

13.1	Waste treatment methods Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

# Contaminated packaging

Dispose of as unused product.

# SECTION 14: Transport information

| 14.1 | UN number |  |
| --- | --- | --- |
| 14.2 | ADR/RID: 1604 UN proper shipping name | IMDG: 1604 |

IATA: 1604

ADR/RID:	ETHYLENEDIAMINE IMDG:	ETHYLENEDIAMINE

IATA:	Ethylenediamine

| 14.3 | Transport hazard class(es) ADR/RID: 8 (3) | IMDG: 8 (3) | IATA: 8 (3) |
| --- | --- | --- | --- |
| 14.4 | Packaging group ADR/RID: II | IMDG: II | IATA: II |

# Environmental hazards

ADR/RID: no	IMDG Marine pollutant: no	IATA: no

# Special precautions for user

no data available

# SECTION 15: Regulatory information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

# Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

# Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

# SECTION 16: Other information

Full text of H-Statements referred to under sections 2 and 3.

Acute Tox.	Acute toxicity

Aquatic Chronic	Chronic aquatic toxicity Flam. Liq.	Flammable liquids

H226	Flammable liquid and vapour.

H302	Harmful if swallowed.

H302 + H332	Harmful if swallowed or if inhaled H311	Toxic in contact with skin.

H314	Causes severe skin burns and eye damage.

H317	May cause an allergic skin reaction.

H332	Harmful if inhaled.

H334	May cause allergy or asthma symptoms or breathing difficulties if inhaled. H412	Harmful to aquatic life with long lasting effects.

Resp. Sens.	Respiratory sensitisation

# Full text of R-phrases referred to under sections 2 and 3

C	Corrosive

R10	Flammable.

R21/22	Harmful in contact with skin and if swallowed.

R34	Causes burns.

R42/43	May cause sensitisation by inhalation and skin contact.

# Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and   is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.